# Voltage Protector Plus

Proteus 9 simulation report

Abdullah Al Masud | AIUB | FSMB Recruitment 2026 Phase 1 | 10 October 2026

# Purpose and circuit

Voltage Protector Plus disconnects a simulated load when voltage is too low, too high or current exceeds its limit. The Proteus design uses an ATmega328P at 16 MHz, a 16x2 LM016L LCD, four buttons and two 5 V relays with 2N2222 drivers and 1N4007 flyback diodes. Two potentiometers emulate 0-300 V and 0-10 A; the green LED represents the load and the red LED indicates FAULT or LOCK.

# Protection and control

Defaults are VMIN 180 V, VMAX 250 V, IMAX 5 A and a 3 s recovery delay. The load starts OFF. Reconnection requires continuously healthy readings, with 5 V voltage and 0.2 A current recovery margins. SET, UP and DOWN edit settings locally; a 9600-baud UART supports STATUS, SET, SAVE, LOG and ACK. EEPROM stores settings and lock reason. The intended load path uses both normally open relay contacts in series.

# Three practical additions

Contact feedback. A2 observes the main switched node. A sustained command/contact mismatch latches LOCK. The backup is commanded OFF before the main, allowing isolation of a stuck-closed main contact when the series circuit operates correctly.

Repeated fault handling. Three distinct ON-to-fault trips within a rolling 60 s window latch LOCK. Second-trip recovery waits at least 10 s. One sustained fault counts once. Safe ACK starts a fresh delay rather than reconnecting immediately.

Event history. LOG reports the latest eight RAM events with timestamps, readings, active limits and cause. This helps explain a shutdown without extra hardware; the history resets at boot.

# Recorded results

Proteus screenshots show local settings, OVERCURRENT OFF in an earlier revision and OVERVOLTAGE OFF. At 252.1 V and 2.00 A, the latest screenshot shows red ON, green OFF and relay levers OFF. For the Proteus firmware profile, 45 host behavior checks and 8 AVR HEX execution checks passed using modeled peripherals; these do not validate the native circuit.

# Scope and next verification

The inputs are low-voltage emulation, without AC RMS sensing or measured physical trip time. Only the main contact has feedback. Native ON-to-OFF transitions, UART, repeated-trip and contact-fault demonstrations still need recorded verification. The supplied Proteus circuit is preserved unchanged; its firmware path must be reselected after extraction.

Evidence: test_evidence/ contains supplied Proteus screenshots, firmware logs and the native demonstration plan.